I2C VIP
Architecture & Verification Document

Verification IP for the I2C Master Controller

| Document Item | Details |
| --- | --- |
| IP / DUT | iic_block |
| Protocol | I2C / IIC Master |
| Host Interface | APB4 Slave |
| External Interface | SDA / SCL pad interface |
| Primary Clock | PCLK |
| Reset | Active-low rst_n |
| Verification Methodology | UVM-based constrained-random and directed verification |
| Simulator / RTL Context | Cadence Xcelium; SystemVerilog RTL |

Revision 1.0  |  27 July 2026

# Contents

- 1. Introduction

- 2. DUT Overview

- 3. VIP Architecture

- 4. Interface and Transaction Model

- 5. Verification Strategy

- 6. Test Plan

- 7. Coverage Plan

- 8. Assertions and Checkers

- 9. Suggested Directory Structure

- 10. Conclusion

# 1. Introduction

This document describes the architecture and verification approach for the I2C Verification IP (VIP) developed for the iic_block I2C Master controller. The VIP is intended to verify the APB-controlled I2C master, including register programming, I2C START/STOP sequencing, address and data transfers, ACK/NACK handling, repeated START, clock stretching, timeout behavior, error recovery, interrupt generation, and automatic/manual operating modes.

The structure follows the organization and intent of the existing AXI4 AVIP architecture document, while keeping the I2C VIP focused and compact for the current DUT scope.

# 2. DUT Overview

The DUT is an APB4 slave-controlled I2C Master. Software or a testbench programs the control and data registers through APB, after which the controller generates the I2C protocol on SDA and SCL. The controller contains a transaction FSM, programmable clock generation, synchronization of pad inputs, timeout/error detection, and interrupt generation.

| Block | Function |
| --- | --- |
| APB4 Register Interface | Provides access to CTRL, STATUS, DATA, ADDR, CLKDIV and TIMEOUT registers. |
| I2C Controller FSM | Executes IDLE → START → address transmit → ACK → data transfer → ACK → STOP. |
| Clock Generator | Generates programmable I2C timing from PCLK using CLKDIV. |
| Pad Synchronization | Synchronizes SDA/SCL inputs and detects line transitions. |
| Error Handling | Detects NACK, SCL timeout, SDA stuck/error conditions and recovers to STOP/IDLE. |
| Interrupt Logic | Generates IRQ on transaction completion and selected error conditions. |

The design specification defines a programmable I2C frequency using IIC_freq = PCLK / (4 × (CLKDIV + 1)). The controller supports manual command execution, automatic mode, and an 8-entry FIFO command mode in the design specification.

# 3. VIP Architecture

The I2C VIP is organized around a transaction-level UVM environment. APB transactions configure and control the DUT, while an I2C target/slave model responds on SDA/SCL. Monitors observe both interfaces and publish transactions to the scoreboard and coverage.

Recommended high-level architecture:

| Component | Responsibility |
| --- | --- |
| APB Master Agent | Drives APB register reads/writes to configure and control the DUT. |
| I2C Slave Agent | Models an I2C target and provides ACK/NACK, read data, clock stretching and fault injection. |
| APB Monitor | Observes APB transfers and reconstructs register activity. |
| I2C Monitor | Decodes START, STOP, address, data, ACK/NACK and repeated START events from SDA/SCL. |
| Scoreboard | Compares expected software/programmed behavior against observed I2C bus activity and DUT status. |
| Virtual Sequencer | Coordinates APB programming and I2C slave responses for complete end-to-end scenarios. |
| Coverage | Measures protocol, register, mode, error, timing and cross coverage. |
| Assertions | Checks APB protocol, I2C timing/line behavior, command handling and status consistency. |

## 3.1 Conceptual Data Flow

APB sequence → APB Master Driver → DUT registers → I2C Controller FSM → SDA/SCL interface → I2C Slave BFM. The APB and I2C monitors publish observed activity to the scoreboard, coverage model and protocol checkers.

# 4. Interface and Transaction Model

## 4.1 APB Interface

| Signal | Dir. | Width | Description |
| --- | --- | --- | --- |
| pclk | I | 1 | APB/system clock |
| rst_n | I | 1 | Active-low reset |
| paddr | I | 12 | Register address |
| psel / penable | I | 1 | APB select and enable phase |
| pwrite | I | 1 | Read/write direction |
| pwdata / prdata | I/O | 32 | Write/read data |
| pready | O | 1 | APB transfer completion |
| pslverr | O | 1 | APB error indication |

## 4.2 I2C Pad Interface

| Signal | Dir. | Description |
| --- | --- | --- |
| sda_in / sda_out / sda_oe | I/O | SDA input, driven value and output enable |
| scl_in / scl_out / scl_oe | I/O | SCL input, driven value and output enable |
| irq | O | Interrupt output |

## 4.3 Key Register Transactions

| Register | Access | Verification Focus |
| --- | --- | --- |
| CTRL | RW | Enable, manual commands, AUTO/FIFO mode, ACK control, command auto-clear |
| STATUS | RO | BUSY, TIP, RXACK, NACK, timeout, SDA error and FIFO status |
| DATA | RW | Transmit and received data behavior |
| ADDR | RW | 7-bit slave address and R/W control |
| CLKDIV | RW | Programmable SCL frequency |
| TIMEOUT | RW | Clock stretching timeout threshold |
| IRQ / IRQ_ADDR | RO / read-clear | Interrupt assertion and clear behavior |

# 5. Verification Strategy

Verification is divided into the following layers:

- Register-level verification: reset values, access permissions, read/write behavior and command auto-clear.

- Protocol-level verification: START, STOP, address, data, ACK/NACK and repeated START.

- Mode-level verification: manual command sequencing, automatic operation and FIFO/burst behavior where implemented.

- Timing verification: CLKDIV frequency programming and clock stretching.

- Error verification: NACK, SCL timeout, SDA error, FIFO overflow and recovery.

- Interrupt verification: completion/error interrupt generation and read-to-clear behavior.

- Structural verification: FSM state reachability and status signal consistency.

# 6. Test Plan

The verification plan contains 25 tests grouped into sanity, manual-mode, automatic-mode, reactive-exception, interrupt and recovery families.

| Tests | Area | Primary Objective |
| --- | --- | --- |
| TC1 | Reset | Reset values, FSM IDLE, BUSY/TIP/IRQ cleared |
| TC2 | Register access | RW/RO behavior and IRQ read-clear |
| TC3–TC5 | Bus control | START, STOP and address transmission |
| TC6–TC9 | Data transfer | 1–4 byte write/read and packing/order |
| TC10–TC11 | ACK handling | ACK success and NACK error response |
| TC12 | Repeated START | Write → repeated START → read |
| TC13 | Clock divider | SCL period follows programmed divider |
| TC14–TC15 | Clock stretching/timeout | Pause/resume and timeout abort |
| TC16 | SDA error | SDA stuck/error detection |
| TC17 | Interrupt | Completion/error IRQ and clear behavior |
| TC18–TC20 | Status bits | BUSY, TIP and RXACK behavior |
| TC21 | Automatic mode | Complete transaction without manual intervention |
| TC22 | Recovery | Next clean transaction succeeds after errors |
| TC23 | Command clear | START/READ/WRITE command bits auto-clear |
| TC24 | FSM | State transition and reachability coverage |
| TC25 | Manual-mode stress | Delayed command sequencing without hang |

## 6.1 Consolidated Regression Structure

| Test File | Coverage Focus |
| --- | --- |
| test_sanity_reset.sv | Reset and APB register access |
| test_manual_mode_matrix.sv | START/STOP/address/data/ACK/status/FSM/manual sequencing |
| test_auto_mode_matrix.sv | Multi-byte, repeated START, clock divider and AUTO mode |
| test_reactive_exceptions.sv | NACK, clock stretching and timeout |
| test_interrupt_standalone.sv | IRQ completion/error and read-clear |
| test_error_recovery_stress.sv | SDA error, sticky status and post-error recovery |

# 7. Coverage Plan

| Coverage Group | Representative Bins |
| --- | --- |
| Register | All valid addresses, RW/RO accesses, reset values, command writes |
| Transaction | Read/write, 1/2/3/4-byte transfers, address values |
| Protocol | START, STOP, repeated START, ACK, NACK |
| Timing | Multiple CLKDIV values, normal and stretched SCL |
| Errors | NACK, timeout, SDA error, FIFO full/overflow |
| Modes | Manual, AUTO and FIFO command execution where applicable |
| Status | BUSY/TIP/RXACK transitions and error flags |
| Interrupt | Completion, error and read-to-clear behavior |
| Cross Coverage | Mode × transfer type; error × transaction phase; length × read/write |

# 8. Assertions and Checkers

The following checks are recommended for the VIP:

- APB transfer legality: PREADY behavior and stable address/control/data during an active transfer.

- Command semantics: command bits are consumed and cleared as specified.

- I2C START/STOP legality: SDA transitions occur while SCL is high.

- SDA stability: SDA remains stable while SCL is high except for START/STOP.

- ACK phase correctness: ACK/NACK is sampled and reflected in RXACK/NACK_ERR.

- Clock stretching: controller does not advance protocol timing while SCL is externally held low.

- Timeout correctness: timeout eventually terminates an overlong stretch and sets SCL_TO.

- Status consistency: BUSY/TIP correlate with active FSM operation.

- Interrupt behavior: IRQ asserts for completion/error and clears through the specified mechanism.

- FSM safety: no illegal/unknown state and all expected transaction paths are reachable.

# 9. Suggested Directory Structure

i2c_vip/
├── doc/
│   └── i2c_vip_architecture_document.docx
├── src/
│   ├── i2c_globals_pkg.sv
│   ├── i2c_if.sv
│   ├── apb_agent/
│   ├── i2c_slave_agent/
│   ├── env/
│   ├── scoreboard/
│   ├── coverage/
│   └── assertions/
├── sequences/
│   ├── apb_sequences/
│   ├── i2c_slave_sequences/
│   └── virtual_sequences/
├── tests/
│   ├── test_sanity_reset.sv
│   ├── test_manual_mode_matrix.sv
│   ├── test_auto_mode_matrix.sv
│   ├── test_reactive_exceptions.sv
│   ├── test_interrupt_standalone.sv
│   └── test_error_recovery_stress.sv
└── sim/
    └── compile/run files

# 10. Conclusion

The proposed I2C VIP provides a compact but complete verification framework for the iic_block master controller. It combines APB programming, an active I2C slave model, protocol monitoring, end-to-end checking, functional coverage and assertion-based validation. The verification plan is aligned with the current DUT specification and the supplied 25-test plan, while retaining a modular structure that can be extended for additional I2C features or future multi-instance integration.